HYDRAMAX

HM1500RECSI

Hydraulically   driven

AC  GENERATOR

Voltage

120 V   or

120/240 V

AC POWER

15000 Watt

FREQUENCY

60 HZ

50 HZ Models

available

Overall  Efficiency                   86 %

Operating Speed              3600 RPM

Minimum Flow                   24  GPM

Ideal Flow                           26 GPM

Maximum Flow                   40 GPM

Free Air Flow                        14  cfm

Recommended  Filteration      10 m

Max.Oil Temperature             180 F

Min.Tank Size                       30 Gal.

Features and Specifications

- Completely Brushless, Maintenance free.

- Pure Sinewave Output.

- Environmentally safe , no Pollution

- Can mount in any Position.

- Hydr.Motor includes Pressure compensating Flow Control. ( Build in )

- Will operate on Open and Closed Systems

- All this in a Package of   13” high , 11” wide and 29” long and weights only  142 Lbs

- The  Hydraulic Requirement is 24 GPM @ 2200 PSI ( other Flow requirements are available )

(845)-292-0800

http://www.getec.com

email         SALES@GETEC.COM

624 Harris Road

Ferndale New York 12734